| # CS&M # Control System & Modeling |  | S.T.I.E Schneider Toshiba Inverter |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modelling Winder/Unwinder Systems |  |  |  |  |  |
| Key words : | Software Architecture |  |  |  |  |
|  | Name |  | Role / Responsibilities |  | Date |
| Issued by : | Jaouher Romdhane |  | R&D Engineer |  | 03/02/2025 |
| Checked by : |  |  |  |  |  |
| Approved by : |  |  |  |  |  |

# Change Record

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 01 | 03/02/2025 | J.Romdhane | Initialized document |

Open issues

Info: To mark questions and open issues just format the text with “Open issue”

Info: To mark questions and open issues just format the text with “Open issue”	2

# Table of Contents

1. Introduction	4

1.1. Objective of the deliverable	4

1.2. Terminology	4

1.3. References	4

2. Control Strategies for Winder/Unwinder Systems	5

2.1. General Design and structure	5

2.2. Winding technique	5

2.3. Web mechanics	6

2.4. Methods of controlling the tension of web	6

2.4.1. Draw control	6

2.4.2. Open Loop Torque Control	6

2.4.2.1. Open Loop Torque Control (for inline Tensioning)	7

2.4.2.2. Open Loop Torque Control with Speed	7

2.4.2.3. Open Loop Torque Control with Speed and Torque Feed Forward	8

2.4.2.4. Torque Follower Control Modes	8

2.4.3. Closed-loop Tension Control	8

2.4.3.1. Tension feedback with Torque (for inline Tensioning)	9

2.4.3.2. Tension feedback with Speed and Torque Feed Forward	9

2.4.4. Closed-Loop dancer roll control	9

3. SE Winder/Unwinder solution	11

3.1. ATV71 with card option VW3A508	11

3.2. TRACTEL Solution	12

4. Competitors Solutions	13

4.1. ABB	13

4.2. Rockwell automation	14

4.3. Siemens	16

4.4. Lenze	18

5. Bibliography	21

# Introduction

## Objective of the deliverable

The goal of this document is to present the current state of art of control strategies for winder/unwinder systems.

## Terminology

| Acronym | Name | Definition |
| --- | --- | --- |
| Web | Web | A web is defined as the material being transported through the machine. It is sometimes referred to as a 'sheet' or 'strip. |
| Strip | Strip | The strip is defined as the material that is being transported through the machine. |
| Section | Section | A Web Handling Machine is broken up into sections. A section consists of one or more drives used to propel the material through the line. |

## References

| Template Documents | Reference / Date |
| --- | --- |

| Applicable Documents | Reference / Date |
| --- | --- |

| Reference Documents | Reference / Date |
| --- | --- |

# Winding Machine

## Introduction

Figure 1: Physical representation of Winder.

The simulation requires the following parameters and variables:

- W: roll width

- r1: core radius

- r2: roll radius

- MT: material thickness

- MV: Material mass per unit volume

- L: Material length

- M: Material mass

- Jr: Material inertia

- Jc: Winder core inertia

- Bw: Winder viscous friction coefficient

This block computes various winder variables using the following equations.

Surface speed:

# Bibliography

[1]	« Tension control_9329-RM002B-EN-E ».